REG NO: 22TD0040   NAME: KEERTHANA M   YEAR/SEM/SEC: Y3/S6/B

SOURCE CODE:

;|||||||||||||||||||||||||||||||||||| |||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

;|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||| |||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

;||	Title		: Buzzer.c 							    ||;

;||	Status		: Completed!						 	    ||;

;||	Features	: ON/Off  buzzer with time  delay intervals		                ||;

;||	Oscillator	 :12MHz 					                            ||;

;||	Enable Buzzer, Put Jumper JP1 to 'E' Mode.                      		    	     ||;

|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

//<<<<<<<<<<<<<<<<<<<<<<	   DIRECTIVES      >>>>>>>>>>>>>>>>>>>>>

#include <LPC214x.h>

#include <stdio.h>

#define BUZZ 7

void Delay(void);

void Wait(void);

//<<<<<<<<<<<<<<<<<<<<<<<<Code Begins Here>>>>>>>>>>>>>>>>>>>>>>>

void Delay()

{

unsigned int i,j;

for(i=0;i<1000;i++)

for(j=0;j<700;j++);

}

void main()

{

PINSEL0	=	0x00;		//Configure Port0.7 as GPIO

IODIR0	=	3 << BUZZ;	//Configure Port0.7 as OutPut pin

while(1)

{

IOSET0 = 1 << BUZZ;

Delay();

IOCLR0 = 1 << BUZZ;

Delay();

}

}

REG NO: 22TD0040   NAME: KEERTHANA M   YEAR/SEM/SEC: Y3/S6/B

SOURCE CODE:

;|||||||||||||||||||||||||||||||||||| ||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

;|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||| ||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

;|	Title		: LCD Demo.c						                |;

;|	Status		: Completed                				    	    |;

;|	Features	: Program to Display Message in 2 lines 		   	    |;

;|	Description	: LCD Used :2x16 LCD in 4-Bit Mode			                |;

;|          Control Lines	:P0.16 (RS) | P0.17 (RW) | P0.18 (EN)         			     |;

;|	Data Lines	:P0.19 - P0.22							     |;

;|          Enable LCD, turn on switch SW28 at LCD label mark position	                 |;

;|||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||| ||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

#include <LPC214x.H>

#define RS	0x10000

#define RW	0x20000

#define EN	0x40000

void lcd_cmd  (unsigned char);

void lcd_data (unsigned char);

void lcd_initialize (void);

void lcd_display (void);

void LCD4_Convert(unsigned char);

const unsigned char cmd[4] = {0x28,0x0c,0x06,0x01};		//lcd commands

unsigned char msg[] = {">PS-Primer 2148<"};			//msg

unsigned char msg1[]= {"::  LCD Demo! ::"};				//msg1

void delay(unsigned int n)

{

unsigned int i,j;

for(i=0;i<n;i++)

{

for(j=0;j<12000;j++)

{;}

}

}

//----------------------------------

//		LCD Command Send

//----------------------------------

void lcd_cmd(unsigned char data)

{

IOCLR0	|= RS;			//0x1000;		//RS

IOCLR0	|= RW;			//0x2000;		//RW

LCD4_Convert(data);

}

void lcd_initialize(void)

{

int i;

for(i=0;i<4;i++)

{

IOCLR0 = 0xF << 19;			//IOCLR 0/1

lcd_cmd(cmd[i]);

delay(15);

}

}

//----------------------------------

//		LCD Data Send

//----------------------------------

void lcd_data (unsigned char data)

{

IOSET0	|= RS;			//0x1000;		//RS

IOCLR0	|= RW;			//0x2000;		//RW

LCD4_Convert(data);

}

//----------------------------------

//		LCD Display Msg

//----------------------------------

void lcd_display (void)

{

char i;

/* first line message */

lcd_cmd(0x80);

delay(15);

i=0;

while(msg[i]!='\0')

{

delay(5);

lcd_data(msg[i]);

i++;

}

delay(15);

/* second line message */

lcd_cmd(0xc0);

delay(15);

i=0;

while(msg1[i]!='\0')

{

delay(5);

lcd_data(msg1[i]);

i++;

}

delay(15);

}

void LCD4_Convert(unsigned char c)

{

if(c & 0x80) IOSET0 = 1 << 22; else IOCLR0 = 1 << 22;

if(c & 0x40) IOSET0 = 1 << 21; else IOCLR0 = 1 << 21;

if(c & 0x20) IOSET0 = 1 << 20; else IOCLR0 = 1 << 20;

if(c & 0x10) IOSET0 = 1 << 19; else IOCLR0 = 1 << 19;

IOSET0	= EN;											//0x4000;		//EN

delay(15);

IOCLR0	= EN;											//0x4000;		//EN

if(c & 0x08) IOSET0 = 1 << 22; else IOCLR0 = 1 << 22;

if(c & 0x04) IOSET0 = 1 << 21; else IOCLR0 = 1 << 21;

if(c & 0x02) IOSET0 = 1 << 20; else IOCLR0 = 1 << 20;

if(c & 0x01) IOSET0 = 1 << 19; else IOCLR0 = 1 << 19;

IOSET0	= EN;											//0x4000;		//EN

delay(15);

IOCLR0	= EN;											//0x4000;		//EN

}

void main()

{

PINSEL1	=	0; 								//Configure P0.16 - P0.31 as GPIO

IODIR0	=	0xFF << 16;	//Configure Pins P0.16 - P0.22 as Output Pins

lcd_initialize();				//Initialize LCD!

lcd_display();				//Display Message in LCD

while(1);

}

REG NO: 22TD0040   NAME: KEERTHANA M   YEAR/SEM/SEC: Y3/S6/B

SOURCE CODE:

;|||||||||||||||||||||||||||||||||||| |||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

;|||||||||||||||||||||||||||||||||||| |||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

;||	Title		: LED Switch.c 						      ||;

;||	Status		: Completed!							      ||;

;||	Features	: Program to read switch status and displayed in point LEDs.    ||;

;||	Description	: Slide Switch	:   P1.24 - P1.31      				      ||;

;|	 LED Port 	:   P1.16 - P1.23  						      ||;

;||          Place jumper J4 ('E' Label position)            					      ||;

;||											      ||;

;|||||||||||||||||||||||||||||||||||| |||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||||;

/**************************************************************************************/

//<<<<<<<<<<<<<<<<<<<Code Begins Here>>>>>>>>>>>>>>>>>>>>>>

#include <LPC214x.h>

#include <stdio.h>

#define LED 16

#define Switch 24

/*****************************     Declarations *********************************/

void Delay(int);

/********************************************************************************/

int main(void)

{

unsigned char Status=1;

PINSEL2	&= 	0xFFFFFFF3;		//Configure P1.16 - P1.31 as GPIO

IO1DIR	=	0x00 << Switch;	//Configure P1.24 - P1.31 as Input

IO1DIR	|=	0xFF << LED;		//Configure P1.16 - P1.23 as Output

IO1PIN	=	0;

while(1)

{

Status = 1;

IOSET1	=	0xFF << LED;

Delay (10);Delay (10);

IOCLR1	=	0xFF << LED;

Delay (10);Delay (10);Delay (10);

while (~Status)

{

Status 	= 	((IO1PIN & (0xFF << Switch)) >> Switch);

IO1PIN	=	((~Status) << LED);

}

}

}

void Delay(int n)

{

int p,q;

for(p=0;p<n;p++)

{

for(q=0;q<0x9990;q++);

}

}

REG NO: 22TD0040   NAME: KEERTHANA M   YEAR/SEM/SEC: Y3/S6/B

SOURCE CODE:

#include <lpc214x.h>		   	//Includes LPC2148 register definitions

#define SW3		1<<24

#define SW4		1<<25

#define SW5		1<<26

#define MAX_BUFFER_SIZE 16

#define DEVICE_ADDR 0xA0

#define BLK_0 0x00

#define BLK_1 0x02

#define MAX_BLOCK_SIZE 256

#define LED1_ON() IO1SET=(1<<16)	//I2C write indicator

#define LED2_ON() IO1SET=(1<<17)	//I2C read indicator

#define LED3_ON() IO1SET=(1<<18)	//Comm failure indicator

#define LED4_ON() IO1SET=(1<<19)	//Comm success indicator

#define LED1_OFF() IO1CLR=(1<<16)

#define LED2_OFF() IO1CLR=(1<<17)

#define LED3_OFF() IO1CLR=(1<<18)

#define LED4_OFF() IO1CLR=(1<<19)

#define Fosc            12000000

#define Fcclk           (Fosc * 5)

#define Fcco            (Fcclk * 4)

#define Fpclk           (Fcclk / 4)

void Delay(unsigned char j);

void Send_Start(void);

void Send_Stop(void);

unsigned char Send_I2C(unsigned char *Data,unsigned char Len);

unsigned char Read_I2C(unsigned char *Data,unsigned char Len);

unsigned char Page_Write(unsigned char BLOCK_NUMBER,unsigned char BLOCK_ADDR,unsigned char *str);

unsigned char Page_Read(unsigned int BLOCK_NUMBER,unsigned char BLOCK_ADDR);

unsigned char I2C_Status(unsigned char status_code);

unsigned char I2C_WR_Buf[MAX_BUFFER_SIZE]={"ARM7 PRIMER BRD "};

unsigned char I2C_RD_Buf[MAX_BUFFER_SIZE];

unsigned char I2C_ER_Buf[MAX_BUFFER_SIZE]={0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0};

unsigned char Status=0;

unsigned char Status_Flag=0;

void  Delay(unsigned char j)

{

unsigned int  i;

for(;j>0;j--)

{

for(i=0; i<60000; i++);

}

}

void Send_Start()

{

I2C1CONSET=0x20;

}

void Send_Stop()

{

I2C1CONSET=0x10;

}

// This function sends sequential data to the EEPROM 24LC04

// The buffer size for EEPROM 24LC04 is 16 bytes

// The Len parameter should not exceed this value

unsigned char Send_I2C(unsigned char *Data,unsigned char Len)

{

while(Len)

{

I2C1DAT=*Data;

if(I2C_Status(0x28))

{

return 1;

}

Len--;

Data++;

}

return 0;

}

// This function reads random data from the EEPROM 24LC04

unsigned char Read_I2C(unsigned char *Data,unsigned char Len)

{

while(Len)

{

if(Len==1)  //Last byte

{

I2C1CONCLR=0x04;    //set hardware to send nack

if(I2C_Status(0x58))	     //last byte has been received and NACK has been returned

{

return 1;

}

*Data=I2C1DAT;

}

else

{

I2C1CONSET=0x04;       //set hardware to send  ack

if(I2C_Status(0x50))	//Byte has been received ACK has been returned

{

return 1;

}

*Data=I2C1DAT;

}

Data++;

Len--;

}

return 0;

}

unsigned char I2C_Status(unsigned char status_code)

{

while(Status_Flag==0);

Status_Flag=0;

if(Status!=status_code)

{

return 1;

}

else

{

return 0;

}

}

unsigned char Page_Write(unsigned char BLOCK_NUMBER,unsigned char BLOCK_ADDR, unsigned char *str)

{

Send_Start();

if(I2C_Status(0x08))	//Start has been transmitted

{

return 1;

}

I2C1DAT=DEVICE_ADDR | BLOCK_NUMBER;	// Send Address

if(I2C_Status(0x18))					//Device address, block num and write has been transmitted

{

return 1;

}

I2C1DAT=BLOCK_ADDR;	// Send block address

if(I2C_Status(0x28))	//Block address has been transmitted

{

return 1;

}

if(Send_I2C(str,MAX_BUFFER_SIZE))	//Send Data

{

Send_Stop();

return 1;

}

Send_Stop();

return 0;

}

unsigned char Page_Read(unsigned int BLOCK_NUMBER,unsigned char BLOCK_ADDR)

{

Send_Start();

if(I2C_Status(0x08))	//Start has been transmitted

{

return 1;

}

I2C1DAT=DEVICE_ADDR | BLOCK_NUMBER;	// Send Address

if(I2C_Status(0x18))	//Device address, block num and write has been transmitted

{

return 1;

}

I2C1DAT=BLOCK_ADDR;

if(I2C_Status(0x28))	//Block address has been transmitted

{

return 1;

}

Send_Start();		 // Repeat Start

if(I2C_Status(0x10))	//Repeated Start has been transmitted

{

return 1;

}

I2C1DAT=DEVICE_ADDR | BLOCK_NUMBER | 0x01; //Device address, block num and read has been transmitted

if(I2C_Status(0x40))	//

{

return 1;

}

if(Read_I2C(I2C_RD_Buf,MAX_BUFFER_SIZE))	//Receive 16bytes of Data from EEPROM

{

Send_Stop();

return 1;

}

Send_Stop();

return 0;

}

void  __irq I2C1_Status(void)

{

Status_Flag=0xFF; 		//update status flag

Status=I2C1STAT;		//Read Status byte

I2C1CONCLR=0x28;

VICVectAddr = 0x00;   	//Acknowledge Interrupt

}

void I2C_Init()

{

PINSEL0|=0x30C00000;

I2C1CONCLR=0x6C;

I2C1CONSET=0x40;

I2C1SCLH  =80;

I2C1SCLL  =70;

/*  Init VIC for I2C1	*/

VICIntSelect = 0x00000000;	    // Setting all interrupts as IRQ(Vectored)

VICVectCntl0 = 0x20 | 19;	   // Assigning Highest Priority Slot to I2C1 and enabling this slot

VICVectAddr0 = (unsigned long)I2C1_Status; // Storing vector address of I2C1

VICIntEnable = (1<<19);

}

void UART0_Init(unsigned int baudrate)

{

unsigned int BAUD_VAL;

BAUD_VAL = (Fpclk/(16*baudrate));

PINSEL0 |= 0x00000005;	        // Enable rx,tx

U0LCR = 0x00000083;	       // 8 bit data,1 stop bit,no parity bit

U0DLL = BAUD_VAL & 0xFF;    // U0DL for low byte

U0DLM = (BAUD_VAL>>8);      // U0DL for high byte

U0LCR = 0x00000003;	     // DLAB =0

}

void UART0_Txmt(unsigned char Chr)  /* Write character to Serial Port    */

{

while (!(U0LSR & 0x20));

U0THR = Chr;

}

unsigned char UART0_Recv(void)   /* Write character to Serial Port    */

{

while (!(U0LSR & 0x01));

return (U0RBR);

}

void UART0_puts(unsigned char *string)

{

while(*string)

UART0_Txmt(*string++);

}

int  main(void)

{

PINSEL2 = 0;

IO1DIR  = (1<<19) | (1<<18) | (1<<17) | (1<<16); // Set P1.16, P1.17, P1.18, P1.19 as Output

IO1DIR &= 0xfC7fffff;

LED1_OFF();LED2_OFF();LED3_OFF();LED4_OFF();

UART0_Init(9600);

I2C_Init();

UART0_puts("\033[2J");

UART0_puts ("********* ARM Primer LPC-2148 I2C EEPROM Demo **********\n\n\r");

UART0_puts (">>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>\n\n\r");

UART0_puts ("[~] Turn SW 20 ON to Write default data to EEPROM! \n\r");

UART0_puts ("[~] Turn SW 21 ON to Read and Display data from EEPROM! \n\r");

UART0_puts ("[~] Turn SW 22 ON to Erase data from EEPROM \n\r");

while(1)

{

if ((IOPIN1 & SW3) == 0) /*...To Load the Default Data to the EEPROM ...*/

{

LED1_ON();  	//Write Indicator

if(Page_Write(BLK_1,0x00,I2C_WR_Buf))

{

UART0_puts("Write Failed");

LED4_ON();

}

Delay(50);

LED1_OFF();

UART0_puts("Data Written Successfully\r\n");

while (!(IOPIN1 & SW3));

}

else if ((IOPIN1 & SW4) == 0)    /*..To Read the Data Stored in the EEPROM...*/

{

LED2_ON();	   //Read indicator

if(Page_Read(BLK_1,0x00))

{

UART0_puts("Read Failed");

LED4_ON();

}

else

{

UART0_puts("Data Read:");

UART0_puts(I2C_RD_Buf);

UART0_puts("\r\n");

}

Delay(50);

LED2_OFF();

while (!(IOPIN1 & SW4));

}

else if ((IOPIN1 & SW5) == 0)

{

LED3_ON();  	//Write Indicator

if(Page_Write(BLK_1,0x00,I2C_ER_Buf))

{

UART0_puts("Write Failed");

LED4_ON();

}

Delay(50);

LED3_OFF();

UART0_puts("Data Erase Successfully\r\n");

while (!(IOPIN1 & SW5));

}

}

}