## Department of Computer Science and Engineering

## Subject Name: COMPUTER ORGANIZATION AND ARCHITECTURE	Subject Code: CS T36

## FACULTY NAME: Ms.B.PABITHA/ Ms. J. ARTHY

## APPROVED BY:

## UNIT – I

BASIC STRUCTURES OF COMPUTER: Functional Units, Multiprocessors and Multicomputers, Memory Locations and Addresses, Memory operations, Instructions and Instruction Sequencing, Addressing modes, Assembly Language, Basic Input/Output operations, Stacks and Queues, Subroutines, Shift and rotate Instructions, Byte-Sorting program.

- What are the functional units?

A computer consists of five functionally independent main parts. They are

- Input

- Memory

- arithmetic and logic

- output and control units

- What is meant by input unit?

Computers accept coded information through input units, which read the data. The most well-known input device is the keyboard. Whenever a key is pressed, the corresponding letter or digit is automatically translated into its corresponding binary code and transmitted over a cable to either the memory or the processor

- What is meant by memory unit?

The function of the memory unit is to store programs and data. There are two classes of storage, called primary and secondary.

Primary storage is a fast memory that operators at electronic speeds. Programs must be stored in the memory while they are being executed.

Memory in which any location can be reached in a short and a fixed amount of timeafter specifying its address is called randam access memory(RAM) the time required to access one word is called memory access time.

- What are the operations in ALU?

Arithmetic and logic operations – Multiplication, division, comparison of numbers etc.

When operands are brought into the processor they are stored in high speed storage elements called Registers. Each Register can store one word of data

- What is meant by output unit?

The output units is the counter part of the input unit. Its function is to sent processed result to the outside world. Output units – Printer(inkjet printer, Laser printer).Graphics display provide both an input function and input function-I/O Unit.

- What are the operations in control unit?

The operation of a computer can be summarized as follows,

- the computer accepts information in the form of programs and data through an input unit and stores it in the memory

- information stored in the memory is fetched, under program control, into an arithmetic and logic unit, where it is processed.

- Processed information leaves the computer through an output unit.

- All activities inside the machines are directed by the control unit.

- Define byte addressability.(Nov 2015)

- Three basic information – bit, byte and word

- A byte Is always 8  bits. But the word length typically ranges from 16 to 64 bits.

- Memory location assignments refer to successive byte locations in memory

- Memory is byte-accessible.

- For 8086, a Word is 16-bits (2 bytes)

- What is meant by multiprocessor?(Nov 2015)

Large computer system may contain a number of processor units, in which case they are called multiprocessor systems.

These systems either execute a number of different application task in parallel, or they execute sub task of a single large in parallel.

- What is meant by multicomputers?(Dec 2014)

The high performance of these systems comes with much increased complexity and cost in contrast to multiprocessor systems, it is also possible to use an interconnected group of complete computer to achieve high total computional power.when the task they are executing need to communicate data, they do so by exchanging messages over a communication network.

This property distinguishes them from shared memory multiprocessors, reading to the name message passing multicomputer.

- What is meant by Big-Endian?

Big-Endian – lower byte addresses are used for the more significant bytes of a word.

- What is meant by little endian?

Little-Endian – lower byte addresses are used for the less significant bytes of a word

- How to align the word?

In the case of 32- bit word length, natural word boundaries occur at addresses 0, 4, 8 as shown in the (fig. byte and word addressing). The word locations have aligned addresses. Words are said to be aligned in memory if they begin at a byte address that is a multiple of the number of bytes in a word.

There is no fundamental reason why words cannot begin at an arbitrary byte address. In that case words are said to have unaligned addresses.

- What are the Ways to indicate the length of the string?

There are two ways to indicate the length of the string.

- The special control character with the meaning “end of string” can be used as last character in the string.

- Processor register can contain a number indicating the length of the string in bytes.

- What are the memory operations?

- To execute an instruction, the processor control circuits must cause the word containing the instruction to be transferred from the memory to the processor

- Two basic operations involving the memory are needed, namely load and store.

- load operation

- store operation

- Define subroutine.(Nov 2015)

Subroutine calls are a special type of branch where we return to one instruction below the calling instruction.

– Provision must be made to save the return address, since it cannot be written into ROM.

- What are the basic I/O operations?

- Program controlled IO

- Memory mapped IO

- Explain stack.

A stack is a list of data elements, usually words or bits, with the accessing restrictions that elements can be added or removed at one end of the list only. This end is called the top of the stack and other end is called the bottom. The structure is sometimes referred to as Push down stack. It uses LIFO principle for two operations-Push and Pop.

- Explain Queue.

Another useful datastructure that is similar to stack is called the queue. Data are stored and retrieved from a queue on FIFO basics. New data are added at the back and retieved from the front..

- What is the difference between stack and queue implementation?

There are 2 important differences between how a stack and a queue is implemented.

One end of the is fixed while the other end rises and falls as  data are pused and poped. A single pointer is needed to point to top of the stack, at on the other hand both ends of the queue move to higher addresses as data are added at the back and removed from the front. So two pointers are needed to keep track of the two ends of the queue.

One way to limit the queue to fixed region in a memory is to use a circular buffer.

- Define Assembly language.

Machine Instructions Are Represented By patterns of 0s and 1s. symbolic names are used to represent patterns. When writing programs for specific computer, such words are normally replaced by acronyms called mnemonics such as MOV,ADD,INC, and BR.R3 is used to refer register 3, and LOC to refer to a location.

A complete set of such symbolic names and rules for their use constitute a programming language, generally referred to as an assembly language.

- What are the basic operations performed by any computer system?	(Nov 2012)

Basic operations of a computer

Inputting

- The process of entering the data and instruction in the computer system.2.

Processing

- Performing arithmetic operations or logical on data to convert them intouseful information3.

Outputting

The process of producing the useful information or result for the user suchas a printed report or visual display4.

Storing

- Storing data and instruction to make them readily available for initial or additional processing whenever required.5.

Controlling

- Directing the manner or sequence in which all of the above operation are performed

- What is syntax of the language?

The set	 of rules for using the mnemonics in the specification of complete instructions and programs is called the syntax of the language.

- What are the instructions does the instructions consists of?

A typical assembly language consists of 3 types of instruction statements that are used to define program operations:

- Opcode mnemonics

- Data definitions

- Assembly directives

- List the various phases of an instruction cycle?			(Nov 2012)

Fectch, Decode & Execute.

- Define Subroutine Nesting.

Subroutine nesting is to have one subroutine call another. In this case, the return address of the second call is also stored in the link register, destroying its previous contents. Hence, it is essential to save the contents of the link register in some other location before calling another subroutines. Otherwise, the return address of the first subroutine will be lost.

- Define The Processor Stack.

The stack pointer points to the stack called the processor stack. The call instruction pushes the contents of the PC on to the processor stack and loads the subroutine address into the PC. The return instruction pops the return address from the processor stack into the PC.

- Define shift and rotate instructions.

There are many applications that require the bits of an operand to be shifted right or left some specified number of bit positions.

The details of how the shifts are performed depend on whether the operand is a signed number or some more general binary coded information. For general operand we use a logical shift , for number we use a arithmetic shift which preserves the sign of the number.

Logical Shift:

Two logical shift instructions are needed , one for shifting left(LShiftL) and another for shifting right (LShiftR).

- What  is meant by subroutine nesting?

Subroutine nesting is to have one subroutine call another. In this case, the return address of the second call is also stored in the link register, destroying its previous contents.

Hence, it is essential to save the contents of the link register in some other location before calling another subroutines. Otherwise, the return address of the first subroutine will be lost.

## 11 MARKS

- Explain the various types of addressing mode with example.(Nov 2015,April 2015)

- Explain the shift and rotate operations with suitable example.(Nov 2015,2014 )

- With a neat diagram explain the functional units of a computer .89.(Nov 2014)

- What are special registers in a typical computer? Explain their purpose with example.(April 2015)

- Describe the organization of stack(4 Marks April 2015)

- Explain with a diagram the design of a fast multiplier using carry save adder circuit.(April 2015)

- Write in detail about functional units of computer.

- Write in detail about basic Input\output operations.

- Explain in detail about multicomputers and microprocessors.

## 10. Write in detail about instruction sequencing.

## UNIT – II

The IA-32 Pentium Example: Registers and Addressing, IA-32 Instructions, IA-32 Assembly Language, Program Flow Control, Logic and Shift/Rotate Instructions, I/O Operations, Subroutines, Other Instructions, Program Examples.

- How to access memory in IA-32 Pentium?

Instructions operate on data operands of 8 or 32 bits (byte and doubleword)

- Little endian

- Multiple byte data operands may start at any byte address (no alignment necessary) IA-32 Registers

- 8x 32-bit general purpose registers

- 8x floating point registers (doubleword or quadword) with extension to 80-bits internally for increased accuracy

- Memory divided into segments and controlled by segment registers

- Instruction pointer = program counter

- Define Intel 32 pentium.

- Intel is by far the most successful computer architecture to date

- Describe Intel architecture (IA) 32bit machines (hence IA-32)

- First IA-32 was the 80386 (1985), then 80486 (1989), Pentium (1993), Pentium Pro (1995),  pentium II (1997), Pentium III (1999), Pentium 4 (2000), Intel Core (2006) GNU Assembler (gas)

- Because GAS was invented to support a 32-bit unix compiler, it uses standard AT&T syntax

- This syntax is neither worse, nor better than the Intel syntax. It's just different.

- When you get used to it, you find it much more regular than the Intel syntax.

- We are going to give lectures using Intel syntax but exercises will use GAS (a real life issue).

- What are the registers used in IA-32 Memory?(Nov 2015,April 2015)

- Status register

- Condition codes CF, ZF, SF, OF

- Program execution mode bits (IOPL, IF, TF) associated with IO and interrupts Register Names

- Registers in early processors map to IA32 registers

- Grouped into data, pointer and index registers

- The E-prefix means a 32-bit version of the register

- We will use this naming convention

- What are the addressing modes in IA-32?(April 2015)

- Immediate ,Direct ,Register,Register indirect.

- Other addressing modes

- Base with displacement

- Base with index and displacement

- What is meant by one byte instructions?

Instructions that only use one register to generate effective address of operand only take 1 byte

  - E.g. INC EDI, DEC ECX

  - Registers specified by 3-biut codes in the single opcode byte

- What are the types of segments used in registers and addressing?(Nov 2014)

– Code segment holds program instructions

– Data segment holds data operands

– Stack segment holds processor stack

- Explain the Assembler directives used in program flow control?

- .data and .code specify start of data and code in the program

- DD allocates 32-bit locations for variables

- MAIN specifies the start of the program

- END indicates the end of the MAIN procedure

- List the CONDITIONAL JUMPS used in program flow control?

DEC ECX

JG STARTADD

- Explain about UNCONDITIONAL JUMP?

JMP ADDR

- One byte or four byte offset forms just list JG

- More powerful addressing modes also allowed e.g. JMP [JUMPTABLE+ESI*4] (index with displacement)

- What is meant  by immediate mode encoding?

MOV EAX,820

  - 5 bytes

  - One byte opcode to specify move operation, that a 32-bit operand is used and the name of the destination register

  - 4-byte immediate value of 820

MOV DL,5

  - 2 bytes as immediate value is 8-bits

- What are the IA-32 Assembly language?

All assembly instructions, assembler directives and macros use the following format:

[label] mnemonic [operands] [; comment]

- What is meant by conditional jumps?

JG STARTADD

- JG relates to results of the most recently executed data manipulation instruction (in this case DEC ECX)

- Jumps if result was > 0 (ECX > 0 in this case)

- Assume STARTADD was 1000 and address after the JG is 1007, relative address is -7 and is stored in the instruction

- Since only one byte is used, jump range is -128 to 127. A 4-byte offset is used if the address is outside this range

- Other jumps such as jump if equal to 0 (JZ or JE), jump if sign bit is set (i.e. result was negative JS) etc

- What is meant by unconditional jumps?(Nov 2014)

JMP ADDR

- One byte or four byte offset forms just list JG

- More powerful addressing modes also allowed e.g. JMP [JUMPTABLE+ESI*4] (index with displacement)

- What are the logic operations?

- AND EBX,EAX

  - E.g. if EAX=0000FFFFH and

- EBX=02FA62CAH what is the result?

  - NOT EBX

- What are the shift and rotate operations?(Nov 2015)

SHL dst,count

- Also SHR, SAL (same as SHL), SAR

- ROL, ROR, RCL, RCR

- What are the shift instructions?

There are four shift instructions:

- SHL		Shift Left Logical

- SHR		Shift right logical

- SAL		Shift left arithmetic (same as SHL)

- SAR		Shift right arithmetic

- Explain the digit packing using examples?

LEA 		EBP,LOC 		EBP points to first byte.

MOV 		AL,[EBP]	 	Load first byte into AL.

SHL 		AL,4 			Shift left by 4 bit positions.

MOV 		BL,[EBP+1]		Load second byte into BL.

AND 		BL,0FH 		Clear high-order 4 bits to zero.

OR 		AL,BL 		Concatenate the BCD digits.

MOV 		PACKED,AL 		Store the result.

- Explain memory – mapped I/O?

LEA 	EBP,LOC 		EBP points to memory area.

READ: 	BT 	INSTATUS,3 		Wait for character to be

JNC 	READ			entered into DATAIN.

MOV 	AL,DATAIN		Transfer character into AL.

MOV 	[EBP],AL		Store the characterin memory

INC 	EBP 			and increment pointer.

ECHO: 	BT 	OUTSTATUS,3 	Wait for display to

JNC 	ECHO			be ready.

MOV 	DATAOUT,AL 	Send characterto display.

CMP 	AL,CR 		If not carriagereturn,

JNE 	READ 			read more characters.

- Explain Isolated I/O?

IN and OUT instructions used only for I/O

- Addresses for these instructions are in a separate address space to other instructions

- IN REG,DADDR and OUT DEVADDR,REF

- REG must be AL or EAX

- 16-bit address space, if 0-255, specified in the opcode

- Otherwise use DX for DADDR e.g. IN REG,DX.

What are the four instructions to push and pop?

There are four instructions for pushing elements onto the stack and for popping them off. The instructions are,

PUSH 		src

POP		dst

PUSHAD

POPAD

- What are the multiply and divide instructions?

The instructions for integer multiply and divide are,

IMUL 		REG,src

IMUL		src

IDIV		src(source operand)

- What are the Multimedia extension (MMX) instructions?

Processing efficiency is achieved if the individual data items,which are usually bytes or 16-bit words, are packed into small groups that can be processed in parallel.

The IA-32 instruction set has a number of instructions that operate in parallel on such data packed into 64-bit quadwords. These instructions are called multimedia extension (MMX) instructions.

Move instructions are provided for transferring 64-bit MMX operands between the memory and the MMX registers. The instruction

PADDB 	MMi,src

- What is meant by vector instructions?

- A set of instruction that are used to perform arithmetic operations on small groups of floating-point numbers is provided.

- SIMD (single – instruction Multiple – data) instructions are useful for vector and matrix calculations in scientific applications.

- What is meant vector dot product program?

- IA-32 program for computing the dot product of two vectors of numbers stored in the memory starting at addresses AVEC and BVEC.

- A scale factor of 4 is used because the vector elements are assumed to be doubleword (4-byte) numbers.

- What is meant by Byte- sorting program?

Register EAX is loaded with the address LIST and serves as a base register in accessing bytes of the list by using the Base with index addressing mode.

Register EDI serves as an index register for the j index in the outer loop, and register ECX serves as an index register for the k index in the inner loop.

Register DL holds the current largest byte as such sublist is traversed.

- What is meant by linked-list insertion and deletion subroutines?

The IA-32 insertion subroutines assumes that the ID number of the new record does not match that of any record already in the list

The IA-32 deletion subroutine assumes that there exists a record in the list with an ID number that matches the contents of RIDNUM.

11 MARKS

- Explain the architecture of IA-32 with s                                                                     suitable block diagram.(Nov 2015)

- Explain the operations IA-32 with example.(Nov 2015,2014)

- Explain the machine instruction format of IA-32.(Nov 2014,April 2015)

- Explain in detail about the various addressing modes available in IA-32 Pentium machine with example.(April 2015)

- Write an Assemble code to check whether the data A is positive or negative number.(April 2015)

- Write an assembly code to check whether the data X is odd parity or even parity data.(April 2015)

- Write in detail about logic and shift instructions of IA-32.

- Discuss in detail about subroutines.

- Briefly explain about (i) Registers (ii) Addressing

- Explain in detail about subroutines of IA-32.

## UNIT – III

INPUT/OUTPUT ORGANIZATION: Accessing I/O Devices, Interrupts, Interrupt Hardware, Enabling and Disabling Interrupts, Handling Multiple Devices, Controlling Device Requests, Exceptions, Use Of Interrupts in Operating Systems, Pentium Interrupt Structure, Direct Memory Access, Busses, Interface Circuits, Standard I/O Interfaces.

- What is meant by Interrupts?

Synchronization is achieved by having the I/O device send a special signal over the bus whenever it is ready for a data transfer operation.

- Name the Two commonly used mechanisms for implementing I/O operations are?

• Interrupts and

• Direct memory access

- What is meant by Direct Memory Access?

For high speed I/O devices. The device interface transfer data directly to or from the memory without informing the processor.

- What is meant by interrupt hardware?

Most computers have several I/O devices that can request an interrupt. A single interrupt request line may be used to serve n devices.

- How to enable and disable interrupts?

All computers fundamentally should be able to enable and disable interruptions as desired. Again reconsider the COMPUTE and PRINT example. When a device activates the interrupt-request signal, it keeps this signal activated until it learns that the processor has accepted its request. When interrupts are enabled, the following is a typical scenario:

• The device raises an interrupt request.

• The processor interrupts the program currently being executed.

• Interrupts are disabled by changing the control bits in the processor status register (PS).

• The device is informed that its request has been recognized and deactivates the interrupt request signal.

• The action requested by the interrupt is performed by the interrupt-service routine.

• Interrupts are enabled and execution of the interrupted program is resumed.

- What are the issues will rise when handle with multiple device?

While handling multiple devices, the issues concerned are:

• How can the processor recognize the device requesting an interrupt?

• How can the processor obtain the starting address of the appropriate routine?

• Should a device be allowed to interrupt the processor while another interrupt is being serviced?

• How should two or more simultaneous interrupt requests be handled?

- What is meant by vectored interrupts?(Nov 2014)

A device requesting an interrupt may identify itself (by sending a special code) directly to the processor, so that the processor considers it immediately.

- What is meant interrupt nesting?(Nov 2015)

The processor should continue to execute the interrupt-service routine till completion, before it accepts an interrupt request from a second device. Privilege exception means they execute privileged instructions. Individual interrupt-request and acknowledge lines can also be implemented. Implementation of interrupt priority using individual interrupt-request and acknowledge lines has been shown.

- What is meant by simultaneous requests?(April 2015)

The processor must have some mechanisms to decide which request to service when simultaneous requests arrive. Here, daisy chain and arrangement of priority groups as the interrupt priority schemes are discussed. Priority based simultaneous requests are considered in many organizations.

- How to control device requests?

At the device end, an interrupt enable bit determines whether it is allowed to generate an interrupt request. At the processor end, it determines whether a given interrupt request will be accepted.

- What are the terms used in exceptions?

The term exception is used to refer to any event that causes an interruption. Hence, I/O interrupts are one example of an exception.

• Recovery from errors – These are techniques to ensure that all hardware components are operating properly.

• Debugging – find errors in a program, trace and breakpoints (only at specific points selected by the user).

• Privilege exception – execute privileged instructions to protect OS of a computer. Use of interrupts in Operating Systems.

- What are the registers in DMA interface?

• Starting address

• Word count

• Status and control flag

- What is meant by bus arbitration?(Nov 2015)

The device that is allowed to initiate data transfers on the bus at any given time is called the bus master. Arbitration is the process by which the next device to become the bus master is selected and bus mastership is transferred to it.

- What are the two approaches in arbitrations?

The two approaches are centralized and  distributed arbitrations.

In centralized, a single bus arbiter performs the required arbitration whereas in distributed, all device participate in the selection of the next bus master.

The bus arbiter may be the processor or a separate unit connected to the bus. The processor is normally the bus master unless it grants bus mastership to one of the DMA controllers.

A simple arrangement for bus arbitration using daisy chain and a distributed arbitration scheme.

- What is meant by scheme in bus?

The Primary function of the bus is to provide a communication path for the transfer of data. It must also look in to,

– When to place information on the bus?

– When to have control signals?

Some bus protocols are set. These involve data, address and control lines. A variety of schemes have been devised for the timing of data transfers over a bus. They are Synchronous and Asynchronous schemes.

- What is meant by synchronous bus?

All devices derive timing information from a common clock line. Equally spaced pulses on this line define equal time intervals. Each of these intervals constitutes a bus cycle during which one data transfer can take place. Timing of an input/output transfer on a synchronous bus.

- What is meant by Asynchronous bus?

This is a scheme based on the use of a handshake between the master and the slave for controlling data transfers on the bus. The common clock is replaced by two timing control lines, master-ready and slave-ready. The first is asserted by the master to indicate that it is ready for a transaction and the second is a response from the slave. The master places the address and command information on the bus. It indicates to all devices that it has done so by activating the master-ready line.

- How to interface circuits?

An I/O interface consists of the circuitry required to connect an I/O device to a computer bus. On one side of the interface, we have bus signals. On the other side, we have a data path with its associated controls to transfer data between the interface and the I/O device – port. We have two types:

1.Serial port and

2.Parallel port

- What are the functions of the I/O interface?

The functions of an I/O interface are:

• Provides a storage buffer for at least one word of data

• Contains status flags that can be accessed by the processor to determine whether the buffer is full or empty

• Contains address-decoding circuitry to determine when it is being addressed by the processor

• Generates the appropriate timing signals required by the bus control scheme

• Performs any format conversion that may be necessary to transfer data between the bus and the I/O device, such as parallel-serial conversion in the case of a serial port

- What are the serial interface circuit involves in?

A serial interface circuit involves – Chip and register select, Status and control, Output shift register, DATAOUT, DATAIN, Input shift register and Serial input/output.

- What are the standard I/O interfaces?

The three major standard I/O interfaces discussed here are:

– PCI (Peripheral Component Interconnect)

– SCSI (Small Computer System Interface)

– USB (Universal Serial Bus)

- What are the limitations of the Port?

Here to add new ports, a user must open the computer box to gain access to the internal expansion bus and install a new interface card. The user may also need to know how to configure the device and the software. And also it is to make it possible to add many devices to a computer system at any time, without opening the computer box.

- What are the characteristics of Device?

The kinds of devices that may be connected to a computer cover a wide range of functionality - speed, volume and timing constraints. A variety of simple devices attached to a computer generate data in different asynchronous mode. A signal must be sampled quickly enough to track its highest-frequency components.

- How to plug – and – play?

Whenever a device is introduced, do not turn the computer off/restart to connect/ disconnect a device. The system should detect the existence of this new device automatically, identify the appropriate device-driver software and any other facilities needed to service that device, and establish the appropriate addresses and logical connections to enable them to communication.

11 MARKS

- Explain the interrupt enabling and disabling mechanism.((Nov 2015)

- Explain the process of DMA with suitable diagram.(Nov 2015,2014,April 2015)

- What is an exception? Explain in detail about the types of exceptions.(Nov 2014)

- Explain the protocols of USB with a neat diagram.(April 2014)

- Describe the hardware mechanism for handling multiple interrupt requests.(April 2015)

- What are handshaking signals? Explain the handshake control of data transfer during input and output operation.(April 2015)

- Explain in detail about Pentium interrupt structure.(April 2015)

- What are the different methods of I/O transfer and explain how is data transfer done using DMA.

- Explain about buses in detail.

- Write short notes on Interrupt and Interrupt Hardware. Write a program that reads a line from the keyboard and echoes it back to display.

## UNIT – IV

THE MEMORY SYSTEM: Some Basic Concepts, Semiconductor RAM Memories, Read-Only Memories, Speed, Size, and Cost, Cache Memories, Performance Considerations, Virtual memories, Memory Management requirements, Secondary Storage.

- What are the techniques to increase the size and speed of the memory?

Several techniques to increase the effective size and speed of the memory:

  - Cache memory (to increase the effective speed).

  - Virtual memory (to increase the effective size).

- What is SRAM cell?

•	Two transistor inverters are cross connected to implement a basic flip-flop.

•	The cell is connected to one word line and two bits lines by transistors T1 and T2

•	When word line is at ground level, the transistors are turned off and the latch retains its state

•	Read operation: In order to read state of SRAM cell, the word line is activated to close switches T1 and T2. Sense/Write circuits at the bottom monitor the state of b and b’

- What is meant by dynamic RAMs?

  - Do not retain their state indefinitely.

  - Contents must be periodically refreshed.

  - Contents may be refreshed while accessing them for reading.

- What is meant by fast page mode?

- Column addresses can be applied to select and place different bytes on the Suppose if we want to access the consecutive bytes in the selected row.

- This can be done without having to reselect the row.

  - Add a latch at the output of the sense circuits in each row.

  - All the latches are loaded when the row is selected.

  - Different data lines.

- Consecutive sequence of column addresses can be applied under the control signal CAS, without reselecting the row.

  - Allows a block of data to be transferred at a much faster rate than random accesses.

  - A small collection/group of bytes is usually referred to as a block.

  - This transfer capability is referred to as the fast page mode feature.

- What is known as memory controller and what are their parts?

- Recall that in a dynamic memory chip, to reduce the number of pins, multiplexed addresses are used.

- Address is divided into two parts:

  - High-order address bits select a row in the array.

  - They are provided first, and latched using RAS signal.

  - Low-order address bits select a column in the row.

  - They are provided later, and latched using CAS signal.

- What is meant by cache memory?

- Processor is much faster than the main memory.

  - As a result, the processor has to spend much of its time waiting while instructions and data are being fetched from the main memory.

  - Major obstacle towards achieving good performance.

- Speed of the main memory cannot be increased beyond a certain point.

- Cache memory is an architectural arrangement which makes the main memory appear faster to the processor than it really is.

- Cache memory is based on the property of computer programs known as “locality of reference”.

- What is meant by locality of reference and what are their types?

- Analysis of programs indicates that many instructions in localized areas of a program are executed repeatedly during some period of time, while the others are accessed relatively less frequently.

  - These instructions may be the ones in a loop, nested loop or few procedures calling each other repeatedly.

  - This is called “locality of reference”.

- Temporal locality of reference:

  - Recently executed instruction is likely to be executed again very soon.

- Spatial locality of reference:

  - Instructions with addresses close to a recently instruction are likely

to be executed soon.

- What is meant by cache hit?

- Existence of a cache is transparent to the processor. The processor issues Read and 
Write requests in the same manner.

- If the data is in the cache it is called a Read or Write hit.

- Read hit:

    - The data is obtained from the cache.

- Write hit:

  - Cache has a replica of the contents of the main memory.

  - Contents of the cache and the main memory may be updated simultaneously.       This is the write-through protocol.

- What is known as cache miss?

- If the data is not present in the cache, then a Read miss or Write miss occurs.

- Read miss:

  - Block of words containing this requested word is transferred from the memory.

  - After the block is transferred, the desired word is forwarded to the processor.

  - The desired word may also be forwarded to the processor as soon as it is  transferred without waiting for the entire block to be transferred. This is called  load-through or early-restart.

- Write-miss:

  - Write-through protocol is used, then the contents of the main memory are      updated directly.

  - If write-back protocol is used, the block containing the addressed word is first brought into the cache. The desired word is overwritten with new information.

- What are all the mapping functions?

- Mapping functions determine how memory blocks are placed in the cache.

- A simple processor example:

  - Cache consisting of 128 blocks of 16 words each.

  - Total size of cache is 2048 (2K) words.

  - Main memory is addressable by a 16-bit address.

  - Main memory has 64K words.

  - Main memory has 4K blocks of 16 words each.

- Three mapping functions:

  - Direct mapping

  - Associative mapping

  - Set-associative mapping.

- What is meant by write buffer?

Write-through:

- Each write operation involves writing to the main memory.

- If the processor has to wait for the write operation to be complete, it slows down the   processor.

- Processor does not depend on the results of the write operation.

- Write buffer can be included for temporary storage of write requests.

- Processor places each write request into the buffer and continues execution.

- If a subsequent Read request references data which is still in the write buffer, then  this data is referenced in the write buffer.

Write-back:

- Block is written back to the main memory when it is replaced.

- If the processor waits for this write to complete, before reading the new block, it is  slowed down.

- Fast write buffer can hold the block to be written, and the new block can be read first.

- What is meant by prefetching?

- New data are brought into the processor when they are first needed.

- Processor has to wait before the data transfer is complete.

- Prefetch the data into the cache before they are actually needed, or a before a Read  miss occurs.

- Prefetching can be accomplished through software by including a special instruction in the machine language of the processor.

  - Inclusion of prefetch instructions increases the length of the programs.

- Prefetching can also be accomplished using hardware:

Circuitry that attempts to discover patterns in memory references and then prefetches according to this pattern.

- What is meant by lockup-Free cache?

- New data are brought into the processor when they are first needed.

- Processor has to wait before the data transfer is complete.

- Prefetch the data into the cache before they are actually needed, or a before a Read  miss occurs.

- Prefetching can be accomplished through software by including a special instruction in the machine language of the processor.

  - Inclusion of prefetch instructions increases the length of the programs.

- Prefetching can also be accomplished using hardware:

Circuitry that attempts to discover patterns in memory references and then prefetches according to this pattern.

- What is meant virtual memories?(Nov 2015,2014)

- Virtual memory is an architectural solution to increase the effective size of the memory system.

- Recall that the addressable memory space depends on the number of address bits in a computer.

For example, if a computer issues 32-bit addresses, the addressable memory space is 4G bytes.

- What is meant by page table, page frame and page table base register?

- Information about the main memory location of each page is kept in the page table.

  - Main memory address where the page is stored.

  - Current status of the page.

- Area of the main memory that can hold a page is called as page frame.

- Starting address of the page table is kept in a page table base register.

- What are the states in processor?

- Processor usually has two states of operation:

  - Supervisor state.

  - User state.

- Supervisor state:

  - Operating system routines are executed.

- User state:

  - User programs are executed.

  - Certain privileged instructions cannot be executed in user state.

  - These privileged instructions include the ones which change page table base register.

  - Prevents one user from accessing the space of other users.

- Explain Electrically Erasable Programmable Read-Only Memory (EEPROM)

  - To erase the contents of EPROMs, they have to be exposed to ultraviolet light.

  - Physically removed from the circuit.

  - EEPROMs the contents can be stored and erased electrically.

- Explain Flash memory?

  - Has similar approach to EEPROM.

  - Read the contents of a single cell, but write the contents of an entire block of cells.

  - Flash devices have greater density.

    - Higher capacity and low storage cost per bit.

  - Power consumption of flash memory is very low, making it attractive for use in equipment that is battery-driven.

  - Single flash chips are not sufficiently large, so larger memory modules are implemented using flash cards and flash drives.

- Explain Programmable Read-Only Memory (PROM)?

  - Allow the data to be loaded by a user.

  - Process of inserting the data is irreversible.

  - Storing information specific to a user in a ROM is expensive.

  - Providing programming capability to a user may be better.

- Explain Erasable Programmable Read-Only Memory (EPROM)?

  - Stored data to be erased and new data to be loaded.

  - Flexibility, useful during the development phase of digital systems.

  - Erasable, reprogrammable ROM.

- Explain about static memories?

- Implement a memory unit of 2M words of 32 bits each.

- Use 512x8 static memory chips.

- Each column consists of 4 chips.

- Each chip implements one byte position.

- A chip is selected by setting its chip select control line to 1.

- Selected chip places its data on the data output line, outputs of other chips are in high impedance state.

- 21 bits to address a 32-bit word.

- High order 2 bits are needed to select the row, by activating the four Chip Select signals.

- 19 bits are used to access specific byte locations inside the selected chip.

- Explain about Dynamic memories?

- Large dynamic memory systems can be implemented using DRAM chips in a similar way to static memory systems.

- Placing large memory systems directly on the motherboard will occupy a large amount of space.

  - Also, this arrangement is inflexible since the memory system cannot be expanded easily.

- Packaging considerations have led to the development of larger memory units known as SIMMs (Single In-line Memory Modules) and DIMMs (Dual In-line Memory Modules).

- Memory modules are an assembly of memory chips on a small board that plugs vertically onto a single socket on the motherboard.

  - Occupy less space on the motherboard.

  - Allows for easy expansion by replacement.

- Define Memory controller.

- Recall that in a dynamic memory chip, to reduce the number of pins, multiplexed addresses are used.

- Address is divided into two parts:

- However, a processor issues all address bits at the same time.

- In order to achieve the multiplexing, memory controller circuit is inserted between the processor and memory.

- How the Address is divided into two parts?

  - High-order address bits select a row in the array.

  - They are provided first, and latched using RAS signal.

  - Low-order address bits select a column in the row.

  - They are provided later, and latched using CAS signal.

- What are the RAID Disk Arrays?(Nov 2014)

- Redundant Array of Inexpensive Disks

- Using multiple disks makes it cheaper for huge storage, and also possible to improve the reliability of the overall system.

- RAID0 – data striping

- RAID1 – identical copies of data on two disks

- RAID2, 3, 4 – increased reliability

- RAID5 – parity-based error-recovery

11 Marks

- Explain the memory hierarchy with respect to speed, size and cost.(Nov 2015)

- Explain the concept of virtual memories.(Nov 2015,2014)

- Discuss the different types of cache mapping techniques that are employed. Point out heir relative advantages and disadvantages.(Nov 2015,2014)

- Discuss in detail about semiconductor RAM memory.

- Explain the organization and accessing of data on a disk.(April 2015)

- Explain how the virtual address is converted into real address in a paged virtual memory system.(April 2015)

- Discuss the different types of ROM.((Nov 2014)

- Explain in detail about secondary storage.

- Discuss in detail about memory management requirements.

- Describe the virtual memory address translation technique.(April 2013)

## UNIT – 5

BASIC PROCESSING UNIT : Some Fundamental Concepts, Execution Of a Complete Instruction, Multiple-Bus Organization, Hardwired Control, Microprogrammed Control, PIPELINING: Basic Concepts, Data Hazards, Instruction Hazards, Influence On Instructions Sets, Datapath and Control Considerations, Superscalar Operations, Performance Considerations

- What are the fundamental concepts of basic processing units?

It can be subdivided into 3 major phases:

1. Fetch cycle

2. Decode cycle

3. Execute cycle

- Sketch the basic instruction cycle?

- What are the operations to be performed for execution of various instructions?

For the execution of various instructions processor has to perform one or more of the following basic operations:

a) Transfer a word of data from one processor register to the another or to the ALU.

b) perform the arithmetic or logic operations on the data from the processor registers and store the result in a processor register.

c) Fetch a word of data from specified memory location and load them into a processor register.

d) Store a word of data from a processor register into a specified memory location.

- Explain the operations of arithmetic or logic operation

Performing an arithmetic or logic operation

 ALU performs arithmetic and logic operations.

 It is a combinational circuit that has no internal memory.

 It has 2 inputs A and B and one output.

 It‟s A input gets the operand from the output of the multiplexer and its B input gets the operand directly from the bus.

 The result produced by the ALU is stored temporarily in register Z.

- How a word can be fetched from memory?

Fetching a word from memory

 To fetch a word of data from memory the processor gives the address of the memory location where the data is stored on the address bus and activates the Read operation.

 The processor loads the required address in MAR, whose output is connected to the address lines of the memory bus.

 At the same time processor sends the Read signal of memory control bus to indicate the Read operation.

 When the requested data is received from the memory its stored into the MDR, from where it can be transferred to other processor registers.

- What actions are necessary  to execute bus instruction?

To execute bus instruction it is necessary to perform following actions:

1. Fetch the instruction

2. Fetch the operand from memory location pointed by R2.

3. Perform the addition

4. Store the results in R1.

- What are all the sequence of control steps required to perform these operations for the single bus architecture?

1.PCout, MARin Yin, select C, Add, Zin

2. Zout, PCin, MARout , MARinM, Read

3. MDRout P,MARin

4. R2out , MARin

5. R2out , Yin,MARout , MARinM, Read

6. MDRout P, select Y, Add, Zin

7. Zout, R1in

- What is The control sequence for unconditional branch instruction ?

1. PCout, MARin, Yin, SelectC, Add, Zin

2. Zout, PCin, MARout, MDRinM, Read

3. MDRoutP,IRin

4. PCout,Yin

5. Offset_field_Of_IRout,SelectY,Add,Zin

6. Zout,PCin

- Explain  hardwired control?(Nov 2015)

The control units use fixed logic circuits to interpret instructions and generate control signals from them.

The fixed logic circuit block includes combinational circuit that generates the required control outputs for decoding and encoding functions.

- Explain about complete processor?

It consists of Instruction unit Integer unit Floating-point unit Instruction cache Data cache Bus interface unit Main memory module Input/Output module.

Instruction unit- It fetches instructions from an instruction cache or from the main memory when the desired instructions are not available in the cache.

Integer unit – To process integer data

Floating unit – To process floating –point data

Data cache – The integer and floating unit gets data from data cache

- Explain about microprogrammed control?

Every instruction in a processor is implemented by a sequence of one or more sets of concurrent micro operations.

Each micro operation is associated with a specific set of control lines which, when activated, causes that micro operation to take place.

Since the number of instructions and control lines is often in the hundreds, the complexity of hardwired control unit is very high.

Thus, it is costly and difficult to design. The hardwired control unit is relatively inflexible because it is difficult to change the design, if one wishes to correct design error or modify the instruction set.

Microprogramming is a method of control unit design in which the control signal memory CM.

The control signals to be activated at any time are specified by a microinstruction, which is fetched from CM.

A sequence of one or more micro operations designed to control specific operation, such as addition, multiplication is called a micro program.

The micro programs for all instructions are stored in the control memory

.

- What are all the components of  microprogrammed control unit?

- control memory

- control address register

- micro instruction register

- microprogram sequencer

- What are the Advantages of Microprogrammed control?(April 2015)

 It simplifies the design of control unit. Thus it is both, cheaper and less error phrone implement.

 Control functions are implemented in software rather than hardware.

 The design process is orderly and systematic

 More flexible, can be changed to accommodate new system specifications or to correct the design errors quickly and cheaply.

 Complex function such as floating point arithmetic can be realized efficiently.

- What are the Disadvantages of microprogrammed control unit?

 A microprogrammed control unit is somewhat slower than the hardwired control unit, because time is required to access the microinstructions from CM.

 The flexibility is achieved at some extra hardware cost due to the control memory and its access circuitry.

- Explain about Microinstruction.

A simple way to structure microinstructions is to assign one bit position to each control signal required in the CPU.

Grouping of control signals

Grouping technique is used to reduce the number of bits in the microinstruction

- What are the signals of microinstruction?

Gating signals: IN and OUT signals

Control signals: Read,Write, clear A, Set carry in, continue operation, end, etc.

ALU signals: Add, Sub,etc

There are 46 signals and hence each microinstruction will have 46 bits.

- Give the difference between horizontal and vertical  organization?

- Give the Advantages of vertical and horizontal organization

1. Vertical approach is the significant factor,it is used to reduce the requirement for the parallel hardware required to handle the execution of microinstructions.

2. Less bits are required in the microinstruction.

3. The horizontal organisation approach is suitable when operating speed of computer is a critical factor and where the machine structure allows parallel usage of a number of resources.

- Give the Disadvantages of vertical and horizontal organization

Vertical approach results in slower operations speed.

Microprogram sequencing

The task of microprogram sequencing is done by microprogram sequencer.

- What are the 2 important factors must be considered while designing the microprogram sequencer?

a) The size of the microinstruction

b) The address generation time.

- What are 3 sources it has during execution of a microprogram the address of the next microinstruction to be executed?

i. Determined by instruction register

ii. Next sequential address

iii. Branch

- Give the Comparison between Hardwired and Microprogrammed Control(Nov 2014)

- Give the basic concepts of pipelining?

Pipelining is a particularly effective way of organization concurrent activity in a computer system.

Instruction pipeline:

The fetch, decode and execute cycles for several instructions are performed simultaneously to reduce overall processing time. This process is referred to as instruction pipelining.

Consider 4 stage process

F  Fetch : read the instruction from the memory

D Decode: Decode the instruction and fetch the some operands

E Execute: perform the operation specified by the instruction.

W Write: Store the results in the destination location.

- Define data hazard?(Nov 2015)

A data hazard is any conditions in which either the source or the destination operands of an instruction are not available at the time expected in the pipeline. As a result some operation has to be delayed ,and the pipeline stalls.

- Explain about Addressing modes(Nov 2014,2015)

Many processors provide various combinations of addressing modes such as index,indirect,auto increment, auto decrement and so on.

We can classify these addressing modes as simple addressing modes and complex addressing modes.

A complex address mode may require several accesses to the memory to reach the named operand while simple addressing mode requires only one access to the memory to reach the named operand.

- What are the Features of modern processor addressing modes.

a) They access operand from the memory in only one cycle.

b) Only load and store instructions are provided to access memory.

c) The addressing modes used do not have side effects.(when a location other than one explicitly named in an instruction as a destination operand is affected, the instruction is said to have a side effect).

- What are The operations the  processor can perform independently?

1. Reading an instruction from the instruction cache.

2. Incrementing the PC.

3. Decoding an instruction by instruction decoder.

4. Reading from and writing into the data cache.

5. Reading the contents of up to two registers from the register file.

6. Writing into one register in the register file.

7. Performing an ALU operation.

- Define superscalar operations?

A processor capable of parallel instruction per cycle is known as superscalar processor. A superscalar processor has multiple execution units (E-units), each of which is usually pipelined, so that they constitute a set of independent instruction pipelines. Its program control unit (PCU) is capable of fetching and decoding several instructions concurrently. It can issue multiple instructions simultaneously. If the processor can issue up to k instructions simultaneously to the various E-units, then k is called instruction issue degree. It can be six or more using current technology.

11 Marks

- Give note on (i) Complete Instruction Execution (ii) Microprogrammed Control(Nov 2015)

- What are data and instructional hazards and how this can be avoided?(Nov 2015)

- Describe the techniques for handling data and instructional hazards in pipelining.(April 2015)

- Explain the basic organization of a microprogrammed control unit and the generation of control signals using microprogram.(April 2015)

- Explain the concept of hardwired control with neat diagram.(Nov 2014)

- What is meant by hazard? Explain the types of hazard with examples.(Nov 2014)

- Explain any two types of multiple processor organizations to speed up the processing.

- Describe the techniques for handling data and instructions hazards in pipeline.(April 2015)

- Explain in detail about superscalar operations.

- Explain in detail about multiple bus organization.