UNIT II

THE IA-32 PENTIUM EXAMPLE

INTRODUCTION

• Intel is by far the most successful computer architecture to date

• Describe Intel architecture (IA) 32bit machines (hence IA-32)

• First IA-32 was the 80386 (1985), then 80486 (1989), Pentium (1993), Pentium Pro (1995),  pentium II (1997), Pentium III (1999), Pentium 4 (2000), Intel Core (2006) GNU Assembler (gas)

• Because GAS was invented to support a 32-bit unix compiler, it uses standard AT&T syntax

• This syntax is neither worse, nor better than the Intel syntax. It's just different.

• When you get used to it, you find it much more regular than the Intel syntax.

• We are going to give lectures using Intel syntax but exercises will use GAS (a real life issue).

REGISTERS AND ADDRESSING

Memory is byte addressable using 32-bit addresses

• Instructions operate on data operands of 8 or 32 bits (byte and doubleword)

• Little endian

• Multiple byte data operands may start at any byte address (no alignment necessary) IA-32 Registers

• 8x 32-bit general purpose registers

• 8x floating point registers (doubleword or quadword) with extension to 80-bits internally for

increased accuracy

• Memory divided into segments and controlled by segment registers

• Instruction pointer = program counter

Segments

– Code segment holds program instructions

– Data segment holds data operands

– Stack segment holds processor stack

• Status register

– Condition codes CF, ZF, SF, OF

– Program execution mode bits (IOPL, IF, TF) associated with IO and interrupts Register Names

fig IA32 Register structure

• Registers in early processors map to IA32 registers

• Grouped into data, pointer and index registers

• The E-prefix means a 32-bit version of the register

• We will use this naming convention

IA-32 ADDRESSING MODES

Examples

• Immediate

– MOV EAX,25 (decimal)

– MOV EAX,3FA00H (the H suffix means hexadecimal)

– NUM EQU 25

MOV EAX,NUM

– MOV EBX,OFFSET LOC (where LOC is an address label)

• Direct

– MOV EAX,[LOC] (brackets not needed if LOC is an address label)

Examples

• Register

– MOV EAX,EBX

• Register indirect

– MOV EAX,[EBX]

• Immediate, direct, register and register indirect are the basic addressing modes.

The ones that follow give more flexibility.

Other addressing modes

• Base with displacement

– MOV EAX,[EBP+60] (word)

– MOV EAL,[EBP+4] (byte)

• Base with index and displacement

– MOV EAX,[EBP+ESI*4+200]

• Have both of these modes as base with displacement can be encoded with 1 less byte

Base with index + disp

Fig examples of addressing mode IA32 architecture

IA-32 Instructions

• MOV dst,src (dst ← [src])

• ADD dst,src (dst ← [dst] + [src])

• SUB dst,src (dst ← [dst] - [src])

• JG LOOPSTART (G means greater than 0)

• MOV EBX,OFFSET LOCATION

– Loads address of label LOCATION into EBX

– What if it is not a fixed address? Use load effective address which is computed dynamically. LEA

EBX,[EBP+12]

Operands

• Note only one operand can be in memory so C ← [A] + [B]

– MOV EAX,A

ADD EAX,B

MOV C,EAX

A simple program

One byte instructions

• Instructions that only use one register to generate effective address of operand only take 1 byte

– E.g. INC EDI, DEC ECX

– Registers specified by 3-biut codes in the single opcode byte

Immediate mode encoding

• MOV EAX,820

– 5 bytes

– One byte opcode to specify move operation, that a 32-bit operand is used and the name of the

destination register

– 4-byte immediate value of 820

• MOV DL,5

– 2 bytes as immediate value is 8-bits

• MOV DWORD PTR [EBP+ESI*4+DISP],10

– DWORD PTR (doubleword) specifies a 32-bit Operation

Displacement fields

• One operand of a two-operand instruction usually a register. Other can be register or

memory

• Two exceptions where both can be in memory

– Source operand is immediate and destination is in memory

– Push/pop

• When both operands are in registers, only one addressing mode byte needed

– ADD EAX,EBX encoded in 2 bytes

– One for opcode and the other for addressing mode (specifies the two registers)

• MOV ECX,N

– 6 bytes

– Opcode

– Addressing mode (specifies direct mode and destination register)

– Four bytes for address N

– A direction bit in the opcode specifies which operand is the source

• ADD EAX,[EBX+EDI*4]

– 3 bytes

– Opcode

– Two addressing mode bytes as two registers used to generate effective address of source operand.

IA-32 ASSEMBLY LANGUAGE

PROGRAM FLOW CONTROL

Assembler directiv es

• .data and .code specify start of data and code in the program

• DD allocates 32-bit locations for variables

• MAIN specifies the start of the program

• END indicates the end of the MAIN procedure

CONDITIONAL JUMPS

DEC ECX

JG STARTADD

– JG relates to results of the most recently executed data manipulation instruction (in this case DEC ECX)

– Jumps if result was > 0 (ECX > 0 in this case)

– Assume STARTADD was 1000 and address after the JG is 1007, relative address is -7 and is stored in the instruction

– Since only one byte is used, jump range is -128 to 127. A 4-byte offset is used if the address is outside this range

– Other jumps such as jump if equal to 0 (JZ or JE), jump if sign bit is set (i.e. result was negative JS) etc

UNCONDITIONAL JUMP

• JMP ADDR

– One byte or four byte offset forms just list JG

– More powerful addressing modes also allowed e.g. JMP [JUMPTABLE+ESI*4] (index with displacement)

LOGIC AND SHIFT/ROTATE INSTRUCTIONS

LOGIC OPERATIONS

AND EBX,EAX

– E.g. if EAX=0000FFFFH and

EBX=02FA62CAH what is the result?

• NOT EBX

SHIFT AND ROTATE OPERATIONS

• SHL dst,count

– Also SHR, SAL (same as SHL), SAR

• ROL, ROR, RCL, RCR

There are four shift instructions:

SHL		Shift Left Logical

SHR		Shift right logical

SAL		Shift left arithmetic (same as SHL)

SAR		Shift right arithmetic

DIGIT PACKING

LEA 		EBP,LOC 		EBP points to first byte.

MOV 		AL,[EBP]	 	Load first byte into AL.

SHL 		AL,4 			Shift left by 4 bit positions.

MOV 		BL,[EBP+1]		Load second byte into BL.

AND 		BL,0FH 		Clear high-order 4 bits to zero.

OR 		AL,BL 		Concatenate the BCD digits.

MOV 		PACKED,AL 		Store the result.

I/O OPERATIONS

MEMORY – MAPPED I/O

LEA 	EBP,LOC 		EBP points to memory area.

READ: 	BT 	INSTATUS,3 		Wait for character to be

JNC 	READ			entered into DATAIN.

MOV 	AL,DATAIN		Transfer character into AL.

MOV 	[EBP],AL		Store the characterin memory

INC 	EBP 			and increment pointer.

ECHO: 	BT 	OUTSTATUS,3 	Wait for display to

JNC 	ECHO			be ready.

MOV 	DATAOUT,AL 	Send characterto display.

CMP 	AL,CR 		If not carriagereturn,

JNE 	READ 			read more characters.

ISOLATED I/O

• IN and OUT instructions used only for I/O

• Addresses for these instructions are in a separate address space to other instructions

• IN REG,DADDR and OUT DEVADDR,REF

– REG must be AL or EAX

– 16-bit address space, if 0-255, specified in the opcode

– Otherwise use DX for DADDR e.g. IN REG,DX.

SUBROUTINES

The processor stack data structure is convenient for handling entry to and return from subroutines. In the IA-32 architecture, register ESP is used as the stack pointer, which points to the current top element (TOS) in the processor stack.

There are four instructions for pushing elements onto the stack and for popping them off. The instructions are,

PUSH 		src

POP		dst

PUSHAD

POPAD

The subroutine is called by the instruction

CALL 		LISTADD

OTHER INSTRUCTIONS

MULTIPLY AND DIVIDE INSRTUCTIONS

The instructions for integer multiply and divide are,

IMUL 		REG,src

IMUL		src

IDIV		src(source operand)

MULTIMEDIA EXTENSION (MNX) INSTRUCTIONS

Processing efficiency is achieved if the individual data items,which are usually bytes or 16-bit words, are packed into small groups that can be processed in parallel.

The IA-32 instruction set has a number of instructions that operate in parallel on such data packed into 64-bit quadwords. These instructions are called multimedia extension (MNX) instructions.

Move instructions are provided for transferring 64-bit MMX operands between the memory and the MMX registers. The instruction

PADDB 	MMi,src

VECTOR INSTRUCTIONS

- A set of instruction that are used to perform arithmetic operations on small groups of floating-point numbers is provided.

- SIMD (single – instruction Multiple – data) instructions are useful for vector and matrix calculations in scientific applications.

PROGRAM EXAMPLES

VECTOR DOT PRODUCT PROGRAM

IA-32 program for computing the dot product of two vectors of numbers stored in the memory starting at addresses AVEC and BVEC.

A scale factor of 4 is used because the vector elements are assumed to be doubleword (4-byte) numbers.

BYTE-SORTING PROGRAM

Register EAX is loaded with the address LIST and serves as a base register in accessing bytes of the list by using the Base with index addressing mode.

Register EDI serves as an index register for the j index in the outer loop, and register ECX serves as an index register for the k index in the inner loop.

Register DL holds the current largest byte as such sublist is traversed.

LINKED-LIST INSERTION AND DELETION SUBROUTINES

Fig An IA-32 subroutine for inserting a new record into a linked list

- Parameters are passed through registers, and register names RHEAD, RNEWREC, RIDNUM, RCURRENT and RNEXT correspond to the same usage as in the generic routines.

- These names have been used instead of the IA-32 names EAX, EBX, and so on.

- A sixth register, RNEWID, has been used to hold the ID number of the new record to be inserted.

- It is loaded with the ID number of the new record by the first instruction of the subroutine.

- The rest of the subroutine is an instruction for instruction match with the subroutine.

- The subroutine for deleting a record, is also an instruction for instruction match with the subroutine

- The IA-32 insertion subroutines assumes that the ID number of the new record does not match that of any record already in the list and the IA-32 deletion subroutine assumes that there exists a record in the list with an ID number that matches the contents of RIDNUM.

Fig An IA-32 subroutine for deleting a record from a linked list