RAJIV GANDHI COLLEGE OF ENGINEERING AND TECHNOLOGY

PONDICHERRY

QUESTION BANK

Subject: - ENGINEERING THERMODYNAMICS

SUBMITTED BY:-

Name: - D. NAGASUNDARAM

Designation: - Assistant Professor

Department: - Mechanical Engineering

RAJIV GANDHI COLLEGE OF ENGINEERING AND TECHNOLOGY

PONDICHERRY

DEPARTMENT OF MECHANICAL ENGINEERING

T105 - ENGINEERING THERMODYNAMICS

UNIT I – BASIC CONCEPTS AND DEFINITIONS

UNIT I –  Energy conversion and efficiencies - system, property and state – Thermal equilibrium – Temperature – Zeroth law of Thermodynamics – Pure substance – P, V and T diagrams – Thermodynamic diagrams.

PART- A

- What is meant by thermodynamic system? How do you classify it? (May 2014)

Thermodynamic system is defined as the any space or matter or group of matter where the energy transfer or energy conversions are studied.

It may be classified into three types.

- Open system

- Closed system

- Isolated system

- What is meant by closed system? Give an example.

When a system has only heat and work transfer, but there is no mass transfer, it is called as closed system.

Example: Piston and cylinder arrangement.

- Define an open system, Give an example.

When a system has both mass and energy transfer it is called as open system.

Example: Air Compressor.

- What is meant by surroundings?

Any other matter outside the system boundary is called as surroundings.

- What is boundary?

System and surroundings are separated by an imaginary line is called boundary.

- Differentiate closed and open system. (Jan 2012)

| Closed System | Open System |
| --- | --- |
| There is no mass transfer. Only heat and work will transfer. | Mass transfer will take place, in addition to the heat and work transfer. |
| System boundary is fixed one | System boundary may or may not change. |
| Ex: Piston & cylinder arrangement, Thermal power plant | Air compressor, boiler |

- Define an isolated system

Isolated system is not affected by surroundings.  There is no heat; work and mass transfer take place.  In this system total energy remains constant.

Example: Entire Universe

- Define: Specific heat capacity at constant pressure.

It is defined as the amount of heat energy required to raise or lower the temperature of unit mass of the substance through one degree when the pressure kept constant.   It is denoted by Cp.

- Define: Specific heat capacity at constant volume.

It is defined as the amount of heat energy required to raise or lower the temperature of unit mass of the substance through one degree when volume kept constant.

- What is meant by thermodynamic property?

Thermodynamic property is any characteristic of a substance which is used to identify the state of the system and can be measured, when the system remains in an equilibrium state.

Thermodynamic property can be classified into two types.

- Intensive or Intrinsic and

- Extensive and Extrinsic property.

- Define Intensive and Extensive properties. (May 2012)

The properties which are independent on the mass of the system is called intensive properties.

e.g., Pressure, Temperature, Specific Volume etc.,

The properties which are dependent on the mass of the system is called extensive properties.

e.g., total energy, Total volume, weight etc.

- Differentiate Intensive and Extensive properties. (Jan 2011)

| Intensive Properties | Extensive Properties |
| --- | --- |
| Independent on the mass of the system | Dependent on the mass of the system. |
| If we consider part of the system these properties remain same. e.g. pressure, Temperature specific volume etc., | If we consider part of the system it will have a lesser value. e.g., Total energy, Total volume, weight etc., |
| Extensive property/mass is known as intensive property | -- |

- What do you understand by equilibrium of a system? (Jan 2011),(May 2013),(May 2015)

When a system remains in equilibrium state, it should not undergo any changes to its own accord.

- What is meant by thermodynamic equilibrium?

When a system is in thermodynamic equilibrium, it should satisfy the following three conditions.

Mechanical Equilibrium:  Pressure remains constant

Thermal equilibrium: Temperature remains constant

Chemical equilibrium: There is no chemical reaction.

- Define the term process.

It is defined as the change of state undergone by a gas due to energy flow.

- Define the term Cycle.

When a system undergoes a series of processes and return to its initial condition, it is known as cycle.

- What is meant by open and closed cycle?

In a closed cycle, the same working substance will re circulate again and again.

In a open cycle, the same working substance will be exhausted to the surroundings after expansion.

- What is meant by reversible and irreversible process? (Jan 2016)

A process is said to be reversible, it should trace the same path in the reverse direction when the process is reversed.  It is possible only when the system passes through a continuous series of equilibrium state.

If a system does not pass through continuous equilibrium state, then the process is said to be irreversible.

- What is meant by Point and Path function? (Jan 2014)

The quantities which are independent on the process or path followed by the system is known as point functions.

Example: Pressure, volume, temperature, etc.,

The quantities which are dependent on the process or path followed by the system is known as path functions.

Example: Heat transfer, work transfer.

- What is Quasi – Static process? (May2012),(Jan2013),(Jan2015)

The process is said to be quasi – static, it should proceed infinitesimally slow and follows continuous series of equilibrium states.  Therefore, the quasi static, it should proceed infinitesimally slowly and follows continuous series of equilibrium states.  Therefore, the quasi static process may be a reversible process.

- Explain Zeroth Law of thermodynamics? (May2013),(May2015),(May2016)

Zeroth law of thermodynamics states that when two systems are separately in thermal equilibrium with a third systems, then they themselves are in thermal equilibrium with each other.

- Define the term enthalpy?

The Combination of internal energy and flow energy is known as enthalpy of the system.  It may also be defined as the total heat of the substance.

Mathematically, enthalpy (H) = U + pv KJ)

Where, U – internal energy

p – Pressure

v – Volume

In terms of Cp & T → H = m Cp (T2-T1) KJ

- Define the term internal energy. (May2016)

Internal energy of a gas is the energy stored in a gas due to its molecular interactions.

It is also defined as the energy possessed by a gas at a given temperature.

- What is meant by thermodynamic work?

It is the work done by the system when the energy transferred across the boundary of the system.  It is mainly due to intensive property difference between the system and surroundings.

- State the law of conservation of energy

Energy can neither be created nor destroyed, but it can be transferred from one form to another.

- What is a pure substance? (May2014)

“The substances of fixed chemical composition are known as pure substances”. The substances may exist in any one at three phases solid, liquid or gas. A pure substance does not have to be a single chemical element or compound. A mixture of various chemical elements or compounds is also called as pure substances.

Example: Water, Oxygen, Nitrogen.

#### Define latent heat of ice.

Total amount of heat added during conversion of 0°C into water of 0°C.

#### Define latent heat of evaporation.

The amount of heat added during heating of water boiling point to dry saturated stage is called as latent heat of vaporization or enthalpy of vaporization or latent heat of steam.

#### Define: sensible heat of water.

The amount of heat required to raise the temperature of unit mass of water from 0°C to the saturation temperature under a constant pressure. It is denoted by hf.

#### Define the term “Super heat enthalpy”.

The heat supplied to the dry steam at saturation temperature, to convert it into superheated steam at the temperature Tsup is called superheat enthalpy.