RAJIV GANDHI COLLEGE OF ENGINEERING AND TECHNOLOGY

DEPARTMENT OF MECHANICAL ENGINEERING

T105 - ENGINEERING THERMODYNAMICS

UNIT II – FIRST LAW OF THERMODYNAMICS

UNIT II – The concept of work and adiabatic process – First law of thermodynamics – conservation of Energy Principle for closed and open systems – Calculation of work for different processes of expansion of gases.

Part - A

- State First law of thermodynamics. (Jan 2011)

First law of thermodynamics for a cycle states that, “when a system undergoes a cyclic process, then the net heat transfer is equal to the net work transfer”.

Q1-2

W1-2

- What is meant by Perpetual motion machine of first kind? (April 2010,Jan 2013)

- A machine which violates the first law of thermodynamics is called PMM-1.

- It is defined as a machine which produces work energy without consuming an equivalent of energy from other source.

- Thus, a PMM 1 is impossible.

Engine

Q

W

Engine

Q

W

(a) PMM 1 (Impossible) 		(b) The converse of PMM1

- Corollaries of first law of thermodynamics. (Jan 2011)

- The exists a property of a closed system such that a change in its value is equal to the difference between the heat supplied and the work done during any change of state.

- The internal energy of a closed system remains unchanged if the system is isolated from its surrounding.

- What are the limitations of first law of thermodynamics? (April 2010)

- According to the first law of thermodynamics it is impossible transfer heat from lower temperature to higher temperature.

- According to the first law of thermodynamics heat and work mutually convertible during any cycle of closed system.

- But this law does not specify the possible conditions under which the heat is converted in to work.

- Define Specific heat capacity.

The specific heat capacity or heat capacity of a substance is defined as the quantity of heat transfer required to raise or lower the temperature of the unit mass of the substance through one degree. Its unit is J/kgK or kJ/kgK.

- Define Specific heat capacity at constant volume.

Specific heat at constant volume of a substance is the amount of heat added to raise the temperature of unit mass of the given substance by one degree at constant volume.

When the gas is heated or cooled at constant volume, the heat transfer is given by,

Q = m Cv (T2 - T1)

Where Q – Heat transferred (kJ),

m – Mass of the gas (kg),

C– Specific heat capacity at constant volume (kJ/kgK),

T1 – Initial temperature of the gas (K),

T2 – Final temperature of the gas (K).

- Define Enthalpy.

- The total heat of substance is known as enthalpy and it is denoted by H.

- Enthalpy is equal to the sum of internal energy and flow energy.

- The specific enthalpy(h) of substance is  a property of the system, where h = u + pv

- Define Specific heat capacity at constant pressure.

Specific heat at constant pressure of a substance is the amount of heat added to raise the    temperature of unit mass of the given substance by one degree at constant pressure.

When the gas is heated or cooled at constant pressure, the heat transfer is given by,

Q = m Cp (T2 - T1)

Where Q – Heat transferred (kJ),

m – Mass of the gas (kg),

Cp – Specific heat capacity at constant volume (kJ/kgK),

T1 – Initial temperature of the gas (K),

T2 – Final temperature of the gas (K).

Cp is always greater than Cv. The ratio of two specific heats (Cp/Cv) remains constant and it is denoted by the symbol γ. It is also called as adiabatic index.

For air, Cp = 1.005 kJ/kgK, Cv = 0.718 kJ/kgK, γ = 1.4.

- Define Internal Energy.

Internal energy of a gas is the heat energy stored in the gas at a given temperature. It is denoted by the letter ‘U’.

U = m Cv T (kJ)

Where m – mass of the gas (kg),

Cv – Specific heat capacity at constant volume (kJ/kgK),

T – Initial temperature of the gas (K)

- What is Change in internal energy?

      - When heat is supplied to a certain mass of gas (a) the temperature of the gas may increase (or) the volume of the gas may increase doing external work (or) (c) it may do both.

      - The increase in heat energy stored in the gas due to raise in temperature is known as increase of internal energy or change in internal energy.

∆U = m Cv (T2 - T1)

Where m – mass of the gas (kg), Cv – Specific heat capacity at constant volume(kJ/kgK), T1 – Initial temperature of the gas (K), T2 – Final temperature of the gas (K).

- What is steady flow process? (Nov 2013, May 2012)

During the process the rate of flow of mass and energy across the boundary remains constant, is known as steady flow process.

- What is Steady flow energy equation?

Consider a thermodynamic system in which a fluid is flowing at a steady state. Fluid enters the system at point 1 and  leaves the system at point 2. The space under consideration between entry and exit is known as control volume.

- Write down the steady flow energy equation.

m (gz1 + C12/2 + u1 + p1v1) + Q = m (gz2 + C22/2+ u2+p2v2) + W

Where         m = mass of the substance.

Z1 & Z2 = Height above the datum levels for inlet and outlet in ‘m’.

u1 & u2 = Specific internal energy of working substance entering and exit of the system.

p1 & p2 = pressures of working substance entering and exit of the system.

v1 & v2 = Specific volume of working substance entering and exit of the system (m3/kg).

C1 & C2 = velocity of working substance entering and exit of the system (m3).

Q = Heat supplied to the system in J/kg.

W = Work delivered by the system in J/kg.

- What is flow energy (or) flow work? (Jan2103)

The energy required to push the fluid into (or) out of the control volume without affecting the values of internal, kinetic and potential energies of the fluid.

- What are the assumptions made in the steady flow system?

- The mass rate of flow through the control volume is constant.

- Only potential, kinetic, internal and flow energy are considered. Other forms of energy are not considered.

- The interaction between the system and the surroundings are only work and heat. The rate at which work and heat cross the boundary is constant.

- The state of fluid at any point remains constant at all times.

- List out the applications of SFEE. (Nov 2013)

SFEE governs the working of a large number of components used in many engineering practices like,

- Steam generators (boilers)

- Steam condensers

- Steam nozzles

- Air compressors

- Steam or gas turbines

- Air heaters etc.

- What is Steam generator or boiler?

Boiler is equipment which generates steam. Water is supplied to the boiler and heated by the   burning fuel. Water is converted in steam. This steam is supplied to the machines for doing work.

Boiler

- No mechanical work is done. Therefore, W = 0.

- Fluid velocity at the inlet and exit is small. There is no change in kinetic energy. Therefore, V1 = V2.

- Potential energy between inlet and exit is also negligible, Z1 = Z2.

- What is function of Steam condenser?

A steam condenser condenses steam into water. The main function of a condenser is to transfer   the heat out of the steady flow system.

- No mechanical work is done. Therefore, W = 0.

- Fluid velocity at the inlet and exit is small. There is no change in kinetic energy. Therefore, V1 = V2.

- Potential energy between inlet and exit is also negligible, Z1 = Z2.

- Define Steam nozzle.

Nozzle is a device used for increasing the velocity of fluid at the cost of pressure drop.

- No mechanical work is done. Therefore, W = 0.

- No heat is transferred. Therefore, Q = 0.

- Potential energy between inlet and exit is also negligible, Z1 = Z2.

Nozzle

- What is steam turbine?

A steam turbine is a device which converts the energy of steam into mechanical work.

- No heat is transferred. Therefore, Q = 0.

- Fluid velocity at the inlet and exit is small. There is no change in kinetic energy. Therefore, V1 = V2.

- Potential energy between inlet and exit is also negligible, Z1 = Z2.

Turbine

- What is the function of Air compressor?

Air compressor is a machine used to produce high pressure air. It takes in atmospheric air and compresses it to a high pressure. It may be broadly classified as (i) rotary compressor and (ii) reciprocating compressor.

- No heat is transferred. Therefore, Q = 0.

- Fluid velocity at the inlet and exit is small. There is no change in kinetic energy. Therefore, V1 = V2.

- Potential energy between inlet and exit is also negligible, Z1 = Z2.

- What are Non-flow thermodynamic processes?

Different types of Non-flow process

- Constant volume (or) isochoric process (V = C)

- Constant pressure (or) isobaric process (p = C)

- Constant temperature (or) isothermal process (T = C or pV = C)

- Adiabatic process (or) Isentropic process (pVγ = C)

- Polytropic process (pVn = C)

- Hyperbolic process (pv = C)

- Free expansion process

- Throttling process

- What is constant volume process? And sketch P-V & T –S diagrams?

T

S

Constant volume heating process,          (b) p-V diagram,                             (c) T-S diagram

An isochoric process, also called a constant volume process, and is volumetric process, or an isometric process, is a thermodynamic process during which the volume of the closed system undergoing such a process remains constant.

The heat supplied during this process is used to increase the pressure and temperature of the gas. All the heat supplied is stored in the gas itself in the form of internal energy.

- What is constant Pressure process? And sketch P-V & T –S diagrams?

For a constant pressure process, the boundary must more against an external resistance as heat is supplied. When a gas is heated at constant pressure, its volume increases and thus work is done.

T

S

p-V diagram  								T-S diagram

- What is Isothermal process? And sketch P-V & T –S diagrams?

T

S

Constant temperature process,             (b) p-V diagram,                  (c) T-S diagram

It is the process during which the temperature (T) remains constant.

- What is Reversible adiabatic (or) isentropic process? And sketch P-V & T –S diagrams?

During adiabatic process, the gas neither receives not rejects heat. i.e. no heat transfer takes place during adiabatic process.

Imagine a gas present inside a cylinder made of insulated walls. The gas in the cylinder may be    compressed or expanded without heat transfer.

- What is Polytropic process? And sketch P-V & T –S diagrams?

Generally many actual processes have expansion and compression curves, which follow the law pVn = C, where n is a constant. The processes (constant volume, constant pressure, constant temperature and adiabatic process) are special cases of polytropic process having different values of n.

P-V diagram for polytropic process.

- What is the formula for the heat transfer equivalent in polytropic process?  (May 2012, Jan 2015)

γ - n

Heat transfer Q =   W

γ – 1

Where γ = Adiabatic index

n = Polytropic index

- Explain free expansion process.   (MAY 2013)

A fluid is allowed to expand suddenly into vacuum chamber through an orifice of large dimension is known as free expansion process. It is also called as unresisting expansion process. It is one of the irreversible processes. Q = 0, W = 0, therefore ΔU = 0.

- Explain throttling process.(MAY 2014)

When a fluid expand through a minute slightly opened walled is called as throttling process.

Q = 0, W = 0, therefore ΔU = 0