RAJIV GANDHI COLLLEGE OF ENGNEERING AND TECHNOLOGY

DEPARTMENT OF MECHANICAL ENGINEERING

T105 - ENGINEERING THERMODYNAMICS

UNIT V – REFRIGERATION CYCLES AND SYSTEMS

UNIT V – Reverse Carnot cycle – COP – Vapor compression refrigeration cycle and systems (only theory) – Gas refrigeration cycle – Absorption refrigeration system – Liquefaction – Solidification (only theory).

Part - A

- Define refrigeration.

Refrigeration is defined as a method of reducing the temperature of a system below that of the surroundings and maintaining it at the lower temperature by continuously extracting the heat from it.

- What is the purpose of refrigerant?

The refrigerant is a heat carrying medium which during their cycle in a refrigeration system  absorbs heat from a low temp. system & delivers it to a higher temperature system.

- Mention the uses of refrigeration.

- Ice making

- Preservation of food products and medicines

- Air conditioning

- Transportation of food stuffs, dry products, flowers etc

- Cooling of concrete for dams

- Treatment of air for blast furnace

- Special industrial processes such as chemicals, medicals, surgical aids, etc

- Processing of food products, beverages, textiles, printing work, etc

- Define COP. (Apr 2010, Jan 2013,May2105)

It is defined as the ratio of heat absorbed by the refrigerant while passing through the evaporator to the work input required to compress the refrigerant in the compressor.

- Define capacity of Refrigerator. (May2102,Jan 2015)

It is defined as the rate at which heat can be removed from the cold body. It is called as the capacity of a refrigerator. It is expressed in terms of Tones of Refrigeration.

- Define one ton of refrigeration.(May 2016)

The capacity of the refrigeration system is expressed in ton of refrigeration which is the unit of refrigeration.

A ton of refrigeration is defined as the quantity of heat absorbed in order to form one ton of ice in 24 hours when the initial temperature of water is zero degree centigrade.

1 ton of refrigeration = 210 kJ/min = 3.5 kW

- Define relative COP.

- It is the ratio of actual COP to the theoretical COP of a refrigerator.

- Actual COP is measured during a test and theoretical COP is obtained by applying the laws of thermodynamics.

- Give the chemical names and boiling point of Refrigerants. (Jan 2011)

- R-10 ---Carbon tetrachloride

- R-11 ---Trichlore monofluoro methane; boiling point = 23.3oC

- R-12 --- Dichloro difluoro methane; boiling point = – 29.8oC

- R-13 --- Mono bromotrifluoro methane; boiling point = 27.8oC

- R-21 --- Dichloro monofluoro methane Ammonia (NH3); boiling point= – 33.3oC

- R-22 --- Monochloro difluoro methane; boiling point = – 41.3oC

- Mention the various methods of producing refrigeration effect.

- Throttling expansion of a liquid with flashing

- Reversible adiabatic expansion of gas

- Irreversible adiabatic expansion (throttling) of real gas

- Thermoelectric cooling

- Adiabatic expansion

- What is the classification refrigeration system? (Nov 2011)

- Ice refrigeration system – Ice is the refrigerants.

- Air refrigeration system – Air is the refrigerants.

- Vapour refrigeration system – ammonia, carbon di oxide, Freon

- Vapour compression refrigeration system

- Vapour absorption refrigeration system

- Special refrigeration system.

- Mixed type

- Write the different types of refrigeration cycles.

- Reversed Carnot cycle

- Vapor compression refrigeration cycle

- Vapor absorption refrigeration cycle

- Bell Coleman cycle or reversed joule cycle

- Aircraft refrigeration cycle

- What is the difference between vapour compression and vapour absorption system refrigeration?

| Sl.no | Vapour Compression | Vapour absorption |
| --- | --- | --- |
| 1. | The system has more wear and tear and produce noise due to moving parts the compressor fan etc.. | Only moving part in the system is aqua pump. Hence the system is quieter in operation and subject to very little wear. |
| 2. | Electric power is needed to drive the system. | Waste or Exhaust steam may be used. No need for electric power. |
| 3. | Capacity of system drops rapidly with lowered evaporator pressure. | Capacity of system decreases very little with lowered evaporative pressure by increasing steam pressure to the generator. |
| 4. | Mechanical energy is supplied through compressor. | Heat energy is utilized. |
| 5. | Charging of refrigerant of the system is easy. | Charging of refrigerant is difficult. |

- Mention the desirable properties of refrigerants.

- It should be non-flammable.

- It should be non-explosive.

- It should be non-toxic.

- It should not react with moisture to form acids.

- It should not contaminate the food stuff or other stored products.

- Write the merits and demerits of Air refrigeration system.

Merits:

- Air is easily available at free of cost.

- Leakage is not a problem.

- Air is non-toxic, non-flammable and hence safe.

- Maintenance cost of the system is very less.

- The air system is light in weight compared to other system.

Demerits:

- The COP of this system is very low in comparison to other systems.

- The weight of air required to be circulated is more compared with refrigerants used in other system. This is due to the fact that heat is carried by air in the form sensible heat.

- What are the assumptions made in vapour compression system?

- Load on evaporation in TOR.

- The temperature required in the refrigerator.

- Atmosphere temperature.

- List out the refrigerants commonly used in practice. (or) Name the any four commonly used refrigerants. (Jan2011, Jan2013)

- Ammonia (NH3).

- Carbon di oxide.(CO2)

- Sulphur di oxide. (SO2)

- Freon-12

- Mention the advantages of vapour compression refrigeration system. (May2013)

- COP is high

- Running cost is less

- For a fixed capacity of refrigeration, it is compact in size

- It can be employed for large range of temperature

- The size of evaporator is small

- It is available in various sizes and capacities with easy control devices

- What are the demerits of vapour compression refrigeration system? (May2013)

- Refrigerant cost is high

- Refrigerant leakage is a problem

- Initial cost is high

- Energy consumption in the form of electricity is high

- Mention the basic components of a vopour compression refrigeration system.

- It consists of five main components, namely

- Compressor

- Condenser

- Receiver

- Throttle valve (or) expansion valve

- Evaporator

- In a vapour compression refrigeration system, where the highest temperature will occur?

(Jan 2011)

The vapour compression refrigeration system, the highest temperatures will accur in after compression.

- In a vapour compression refrigeration system, where the lowest temperature will occur?                      (May 2011)

The vapour compression refrigeration system, the lowest temperatures will accur in at the inlet of evaporator.

- What are the advantages of vapour absorption refrigeration system?

- A liquid is compressed instead of vapour and thus the work input required is very less

- It has the same COP at part loads as well as the full loads

- Absorption systems are silent in operation

- It is suitable for large capacities

- Operation and maintenance cost is less.

- What are the disadvantages of vapour absorption refrigeration system?

- The COP of the system is less

- It is not economical for small capacity systems

- It need more space than vapour compression refrigeration system

- Charging of the system is difficult.

- List out the applications of cryogenics.

- In preservation of biological specimens

- Liquefaction of gases

- Production of dry ice, i.e solid CO2

- Cryogenic engine for space application

- Cooling system for supercomputers

- Cooling system for electronic components

- What are factors to be considered in air conditioning room? (or) In which factor the performance of refrigeration system measured? (May 2011)

- Temperature of air.

- Humidity of air.

- Purity of air.

- Motion of air.

- Define Liquefaction of gases.

An important area of refrigeration is the liquefaction of gases since many engineering processes at temperature below -100oC (Cryogenic temperature) depends on the liquefied gases. Eg. Separation of Oxygen and Nitrogen from air, Preparation of liquid propellants for rockets.

- Define volumetric efficiency.

It is defined as the ratio of actual volume of gas drawn in to the compressor (at evaporator temperature and pressure) on each stroke to the piston displacement.

- List out the comparisons of Linde and Claude cycles.

| - Sl.No. | - Linde cycle | - Claude cycle |
| --- | --- | --- |
| - 1 | - Less efficient | - More efficient |
| - 2 | - Does not use an expander | - Uses an expander |
| - 3 | - Low temperature achieved | - Very low temperature achieved |
| - 4 | - One heat exchanger is used | - Double heat exchanger is used |
| - 5 | - Dual pressure Linde system can be used | - Dual pressure system cannot be used |
| - 6 | - Used for liquefaction of air, hydrogen | - Used for liquefaction of air, helium |
| - 7 | - Lesser equipment, cheaper to implement | - More equipment, costlier to implement |

- Sketch the T-S and P-V diagram for reversed Carnot cycle.

P-V & T-S diagram for Reversed Carnot Cycle

- Process 1-2 = Adiabatic compression

- Process 2-3 =  Constant Temperature heat rejection

- Process 3-4 = Adiabatic Expansion

- Process 4-1 = Constant Temperature heat addition

- Sketch the T-S and P-h diagram for Vapour compression cycle.

T-s and p-h diagrams for an ideal vapor-compression refrigeration cycle

- Process 1-2 = Adiabatic compression

- Process 2-3 = Constant Temperature heat rejection

- Process 3-4 = Adiabatic Expansion

- Process 4-1 = Constant Temperature heat addition